| Type of Assessment | HAZID ONLY |  | ☐ | HIRA ONLY | ☐ | Risk Assessment |  |  | ☒ | Aspects and Impacts | ☐ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Equipment Spread | HAUV3 |  |  |  |  |  | Ref. No. | 23-0022-HSEQ-RA-001 |  |  |  |
| Task Details | Offshore Survey of OGGS Pipeline and EA Field |  |  |  |  |  | Location: | EA Field, Offshore, Nigeria |  |  |  |
| MODUS Manager / Supervisor |  | Anthony Brown |  |  |  |  | Position | Project Manager |  |  |  |

| RA Development / Review Team |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Andrew Millichap-Bell | Position | HSEQ Manager | Company | MODUS |
| Name | Garry Comb | Position | AUV Manager | Company | MODUS |
| Name | Anthony Brown | Position | Project Manager | Company | MODUS |
| Name |  | Position |  | Company |  |

| Reference Documents (such as Lift Plans, Task Plans applicable to the task(s)) |  |
| --- | --- |
| Sabertooth Technical Manual 40-4162700 | AUT-HV-EOP-001, HAUV Battery Emergencies |
| Task Plan – Deconstruction, Reconstruction and Charging of HV Battery | HS-FM-018 PTW Proforma |
| AS-RA 001- Use of Hand Tools | AS-RA -002 Use of Power Tools |
| AS-RA-003 Hot Work | AS-RA-004 Manual Handling |
| AS-RA-010 Use of Pneumatic Tools | AS-RA-012 Use of Compressor |
| AS-RA-008 Working at Height | HS-PR-007, Workplace Inspections |
| HS-PR-003, Risk Management | HS-PR-011, Lifting Equipment Operations |
| HS-PR-019, Manual Handling | HS-PR-013, PPE |
| AS-RA-015, HV Battery Handling |  |

| Reason for RA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New Task | ☒ | Review | ☐ | MOC | ☐ | Other (please specify below) | ☐ |

| Date of RA | 26.07.2024 | Review Date | Prior to task commencement |
| --- | --- | --- | --- |

|  | POTENTIAL CONSEQUENCE |  |  |  |  | PROBABILITY |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SEVERITY RATING | PEOPLE | ENVIRONMENT | ASSETS WORKSITE / PROPERTY / PLANT / EQUIPMENT | COMMERCIAL COMPANY & CLIENT REPUTATION | COST OF LOSS / IMPACT | VERY LIKELY (5) | LIKELY (4) | POSSIBLE (3) | UNLIKELY (2) | VERY UNLIKELY (1) |
| Catastrophic (5) | Fatalities / Multiple Serious Injuries / Long Term Health Effect | Catastrophic Environmental Impact (Widespread) | Catastrophic Asset Damage | Catastrophic Disruption – International Media Coverage | > £1,000,000 | 25 | 20 | 15 | 10 | 5 |
| Serious (4) | Lost Time Incident Injury / Medium Term Health Effect | Serious Environmental Impact (Localised) | Serious Asset Damage | Serious Disruption – National Media Coverage | >£500,000 | 20 | 16 | 12 | 8 | 4 |
| Moderate (3) | Restricted Work Case / Short Term Health Effect | Moderate Environmental Impact | Moderate Asset Damage | Moderate Disruption – National Media Coverage | >£250,000 | 15 | 12 | 9 | 6 | 3 |
| Minor (2) | Medical Treatment Required / Slight Health Effect | Minor Impact / Spill > 1 ltr | Minor Asset Damage | Minor Disruption – Localised Media Interest | >£50,000 | 10 | 8 | 6 | 4 | 2 |
| Negligible (1) | First aid, no LTI | Negligible Impact / Spill < 1 ltr | Negligible Asset Damage | Negligible Disruption – No Media Interest | <£50,000 | 5 | 4 | 3 | 2 | 1 |

| Risk Acceptability |  |  |
| --- | --- | --- |
| Low Risk | 1 -5 | May proceed without additional controls |
| Medium Risk | 6 – 12 | Additional consideration and risk reduction required. |
| High Risk | 15 - 25 | May not proceed. Alternative method required. |

| RA Register |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Task / Activity | Hazard / Aspect | Consequence / Impact | Severity | Probability | Risk Rating | Control Measures | Severity | Probability | Risk Rating | # Actioned By |
| Risk Assessment Communication |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Communication of 23-0022 Operations risk assessment | Potential lack of understanding risks and control measures | Personal injury or fatality | 5 | 3 | 15 | Risk Assessment shall be communicated to appliable personnel (and signed to confirm understanding). | 5 | 1 | 5 | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Toolbox Talk delivered to applicable personnel (incl. sub-contractors). Documented and attendance recorded. |  |  |  |  |
| 2 | Review of 23-00222 Operations risk assessment | Potential for ineffective risk evaluation | Personal injury or fatality | 5 | 3 | 15 | Risk assessment shall be reviewed upon identification of change to location facilities; work activities; and/or specific personal requirement(s). | 5 | 1 | 5 | Facilitated by HAUV Supervisor (with assistance from PE and HSEQ) |
| Working at Quay / Port Edge |  |  |  |  |  |  |  |  |  |  |  |
| 3 | Operations from quay / port edge | Potential to fall into water | Personnel injury or fatality (drowning) | 5 | 3 | 15 | Work area restricted with no unauthorised access permitted | 5 | 1 | 5 | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Lift jacket shall be available and mandatory 2m from edge |  |  |  |  |
|  |  |  |  |  |  |  | Lift Ring shall be available and instruction for use |  |  |  |  |
|  |  |  |  |  |  |  | Toolbox talk delivered and recorded prior to activities |  |  |  |  |
|  |  |  |  |  |  |  | Personnel empowered to call an ‘All Stop’ if unidentified hazards are observed |  |  |  |  |
|  |  | Movement of vehicles and pedestrians | Personnel injury by being struck by vehicle | 4 | 3 | 12 | Vehicle / Plant restrictions implemented during operations | 4 | 1 | 4 | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Traffic Management arrangements – separation from pedestrian areas |  |  |  |  |
|  |  |  |  |  |  |  | Speed limit enforced |  |  |  |  |
|  |  |  |  |  |  |  | Reverse parking implemented and enforced |  |  |  |  |
|  |  |  |  |  |  |  | Use of Banksman |  |  |  |  |
|  |  |  |  |  |  |  | Trained and competent personnel |  |  |  |  |
|  |  |  |  |  |  |  | Transportation of main items during quiet periods (where possible) |  |  |  |  |
|  |  |  |  |  |  |  | Site induction to include interface with vehicles |  |  |  |  |
|  |  |  |  |  |  |  | Correct PPE mandatory, incl. Hi-Vis |  |  |  |  |
|  |  |  |  |  |  |  | Call All Stop if unidentified hazards are observed |  |  |  |  |
| Environmental Risks |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Offshore Operations | Activities of subsea assets | Marine environment (seabed and associated flora / fauna) | 4 | 3 | 12 | Operator competence / control of asset will reduce impacts on marine life and vegetation | 4 | 1 | 4 | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Pre and Post Diving Checks implemented |  |  |  |  |
|  |  |  |  |  |  |  | Planned maintenance performed |  |  |  |  |
|  |  |  |  |  |  |  | Operated within authorised areas only |  |  |  |  |
|  |  |  |  |  |  |  | Compliance with permits and consents |  |  |  |  |
|  |  | Chemicals and liquids handling, storage and disposal | Potential marine / coastal pollution Potential toxic effects on marine plant and wildlife | 3 | 3 | 9 | Control of Hazardous Substances procedure adhered | 3 | 1 | 3 | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Bridging documents to adequately link the vessel and Contractor processes |  |  |  |  |
|  |  |  |  |  |  |  | Designated storage areas with bunding or similar implemented |  |  |  |  |
|  |  |  |  |  |  |  | Chemicals stored securely and locked in cabinet when not in use |  |  |  |  |
|  |  |  |  |  |  |  | Chemical risk assessments, incl. use, storage and disposal controls for each substance |  |  |  |  |
|  |  |  |  |  |  |  | Regular work inspections implemented and hazards reported and rectified |  |  |  |  |
| Equipment Set-Up |  |  |  |  |  |  |  |  |  |  |  |
| 5 | Manual Handling | Potential for lifting equipment and exposure to sharp edges and finger traps (nips) | Personal injury | 3 | 3 | 9 | Refer to risk assessment: AS-RA-004 Manual Handling | 3 | 2 | 6 | Facilitated by HAUV Supervisor |
| 6 | Use of Hand Tools | Potential for: - Misuse of tools for task - Failure of tools during task | Personal injury | 3 | 3 | 9 | Refer to risk assessment: AS-RA 001- Use of Hand Tools | 3 | 1 | 3 | Facilitated by HAUV Supervisor |
| 7 | Use of Power Tools | Potential for: - Interface with rotating machinery - Contact with electrical supply - Contact with sharp edges and steel shards / splinters - Inhalation of dust - Noise exposure | Personal injury / ill-health | 4 | 3 | 12 | Refer to risk assessment AS-RA -002 Use of Power Tools | 4 | 1 | 4 | Facilitated by HAUV Supervisor |
| 8 | Performing Hot Work | Potential for: - Exposure to shocks or electrocution - Flying sparks - Exposure to types of non-ionizing radiation - Ignition and contact through heat conduction - Exposure to dangerous fumes | Personal injury / ill-health | 4 | 3 | 12 | Refer to risk assessment: AS-RA-003 Hot Work | 4 | 1 | 4 | Facilitated by HAUV Supervisor |
| 9 | Working at Height | Potential for: - Poor planning - Equipment failure - Incorrect technique - Ladder failure | Personal injury / fatality | 5 | 3 | 15 | Refer to risk assessment: AS-RA-008 Working at Height | 5 | 1 | 5 | Facilitated by HAUV Supervisor |
| 10 | Use of Pneumatic Tools | Potential for: - Unfamiliarity with the tools - Contact with metalwork fluids - Slip and trips within work location - Contact with flying debris - Exposure to Hand-Arm Vibration | Personal injury / ill-health | 4 | 3 | 12 | Refer to risk assessment: AS-RA-010 Use of Pneumatic Tools | 4 | 1 | 4 | Facilitated by HAUV Supervisor |
| 11 | Use of Air Compressor | Potential for: - Unfamiliarity with the Air Compressor - Body contact with Compressor - Slip and trips within work location - Contact with stored energy - Electric shock | Personal injury | 4 | 3 | 12 | Refer to risk assessment: AS-RA-012 Use of Compressor | 4 | 1 | 4 | Facilitated by HAUV Supervisor |
| 12 | Use of Subsea Acoustic Measuring Equipment | Potential for: - Fall into water (whilst deploying subsea) - Electrical shock from damage to cable / wiring configuration - Slip and trips within work location | Personal injury | 4 | 3 | 12 | Equipment deployed and configured by Competent Person | 4 | 1 | 4 | Managed by FADFAE (Client) |
|  |  |  |  |  |  |  | Acoustic equipment certified for use and calibrated |  |  |  |  |
|  |  |  |  |  |  |  | Visual inspection on wiring and cable coils to detect any damage prior/post each use. Note repair kit to be available for use if required. |  |  |  |  |
|  |  |  |  |  |  |  | Ballast to be added to the cabling (if required) to ensure drops to required depth and prevent person over-reaching from vessel. |  |  |  |  |
|  |  |  |  |  |  |  | Cable management and segregation to be implemented. Note daily worksite inspections to be implemented |  |  |  |  |
| 13 | Lifting Operations | Potential for lift failure. Potential for dropped and swinging loads | Personal injury / fatality | To be included in FADFAE risk evaluation |  |  | Lifting equipment and accessories provision by FADFAE | To be included in FADFE risk evaluation |  |  | Managed by FADFAE (Client) |
|  |  |  |  |  |  |  | Lifting Operations for mobilisation and demobilisations performed by FADFAE |  |  |  |  |
|  |  |  |  |  |  |  | Lifting Operations for HAUV launch and recovery managed and performed by FADFAE |  |  |  |  |
|  |  |  |  |  |  |  | Modus to provide technical support for Modus equipment positioning and configuration |  |  |  | Facilitated by HAUV Superintendent |
| Work on Electrical Systems |  |  |  |  |  |  |  |  |  |  |  |
| 14 | General electrical tasks | Potential for contact with electrical current | Personal injury through electrical shock / burns | 4 | 3 | 12 | Competent personnel only shall perform electrical work | 4 | 1 | 4 | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Isolation procedure to be followed |  |  |  |  |
|  |  |  |  |  |  |  | Portable electrical equipment to be in receipt of current PAT test sticker |  |  |  |  |
|  |  |  |  |  |  |  | Equipment and cables subject to visual inspection prior to use |  |  |  |  |
|  |  |  |  |  |  |  | PUWER assessment performed on installation completion |  |  |  |  |
|  | High Voltage electrical tasks | Potential for contact with electrical current | Personal injury / fatality through electrical shock | 5 | 3 | 15 | Personnel performing HV works shall be trained and competent | 5 | 1 | 5 | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Isolation procedure to be followed |  |  |  |  |
|  |  |  |  |  |  |  | Permit to Work (HV) issued and monitored |  |  |  |  |
|  |  |  |  |  |  |  | HV gloves available and to be worn (and in date) |  |  |  |  |
|  |  |  |  |  |  |  | Work area restricted with authorised personnel only |  |  |  |  |
|  |  |  |  |  |  |  | Workplace inspection prior to task – defects reported immediately |  |  |  |  |
| 15 | Deconstruction, Reconstruction and charging of battery | Potential for lithium-polymer battery contact. Potential for lithium -polymer battery fire release. | Personal injury / fatality through contact with live parts. Personal injury / fatality through exposure to battery fire and fume release | 5 | 3 | 15 | Refer to risk assessment: AS-RA-015, HV Battery Handling | 5 | 1 | 5 | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Refer to Task Plan: AS-TP-001, Task Plan – HV Battery Assembly and Disassembly |  |  |  |  |
|  |  |  |  |  |  |  | Refer to procedure: AUT-HV-EOP-001, HAUV Battery Emergencies |  |  |  |  |
| 16 | HV battery exposure to high temperature | HV battery overheating from exposure to high water temperature | Potential for release of asphyxiant gas, toxic substances and runaway exothermic reaction | 5 | 2 | 10 | Battery adverse reaction whilst submerged in water shall reduce likelihood of fire (cooling effect) and reduce concentration of hazardous gases | 5 | 1 | 5 | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | HAUV shall be observed for any deficiencies upon recovery to vessel. Note in event of smoke and/or fume release – shall remain in water until deemed safe to recover. |  |  |  |  |
|  |  |  |  |  |  |  | Battery temperature monitoring for duration of project via tether attachment |  |  |  |  |
|  |  |  |  |  |  |  | Temperature readings to be logged in DPR for project duration |  |  |  |  |
|  |  |  |  |  |  |  | HAUV shall be operated in accordance with: Sabertooth Technical Manual 40-4162700 |  |  |  |  |
|  |  |  |  |  |  |  | Refer to Modus document: AUT-HV-EOP-001, HAUV Battery Emergencies |  |  |  |  |
|  |  | HV battery overheating from exposure to direct extreme high temperature | Potential for fire, release of asphyxiant gas, toxic substances and runaway exothermic reaction | 5 | 3 | 15 | Battery temperature monitoring for duration of project via tether attachment | 5 | 1 | 5 | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Temperature readings to be logged in DPR for project duration |  |  |  | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Deck layout to include secure and robust tarpaulin cover to provide shelter for HAUV (and personnel working on it). Note HAUV and integral equipment shall not be exposed to direct sunlight for prolonged period. |  |  |  | Managed by Vessel Master (under control of FADFAE - Client) |
|  |  |  |  |  |  |  | HAUV shall be stored in accordance with: Sabertooth Technical Manual 40-4162700 |  |  |  | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Regular water hosing of HAUV whilst on deck. Note to increase frequency subject to temperature readings / findings. |  |  |  | Hosing equipment to be provided / maintained by FADFAE (Client) |
|  |  |  |  |  |  |  |  |  |  |  | Hosing activity facilitated by Modus Supervisor |
|  |  |  |  |  |  |  | Refer to Modus document: AUT-HV-EOP-001, HAUV Battery Emergencies |  |  |  | Facilitated by HAUV Supervisor |
| Garage (with HAUV docked) |  |  |  |  |  |  |  |  |  |  |  |
| 17 | Deployment of Garage subsea | Potential for Garage to spin when lowering / recovering to vessel. | Equipment damage due to vehicle tether entanglement | 3 | 3 | 9 | HAUV deployment depth is circa 30m water depth which helps reduce the possibility of the Garage spin, | 3 | 1 | 3 | Managed by FADFAE (Client) Note technical assistance facilitated by Modus Superintendent |
|  |  |  |  |  |  |  | The A-frame is designed so the Garage can be latched in any orientation during recovery. |  |  |  |  |
|  |  |  |  |  |  |  | Non-rotating deployment rope to be used for Garage deployment. (Client supply) |  |  |  |  |
|  |  |  |  |  |  |  | Garage will naturally weathervane into a current due to the larger surface area (wings) at the rear. |  |  |  |  |
|  |  |  |  |  |  |  | HAUV able to use thrusters to dampen any Garage swing that occurs and make small heading changes. |  |  |  |  |
|  |  |  |  |  |  |  | System tether can be managed to allow slack to accumulate on seabed. |  |  |  |  |
|  |  |  |  |  |  |  | In event of strong currents – HAUV can be allowed to drift during deployment to reduce effects of current on the Garage during deployment |  |  |  |  |
|  |  |  |  |  |  |  | In event of further control - a tag-line can be attached to the front of the Garage and controlled via a forward mooring fairlead on vessel for deployment. |  |  |  |  |
| 18 | Access to HAUV for maintenance / repair | Potential for HAUV fall / tilt from height | Personal injury / fatality | 5 | 3 | 15 | The subsea garage has also been modified so that the top half of the garage is detachable. This allows the offshore team to remove the top section of the garage with HAUV3 inside, providing dual use, as the garage becomes a work stand for maintenance when required. | 1 | 5 | 5 | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Refer to risk assessment: AS-RA-008 Working at Height |  |  |  |  |
|  |  |  |  |  |  |  | Refer to risk assessment: AS-RA 001- Use of Hand Tools |  |  |  |  |
|  |  |  |  |  |  |  | Refer to risk assessment AS-RA -002 Use of Power Tools |  |  |  |  |
|  |  |  |  |  |  |  | Maintenance activities to be performed in accordance with: Sabertooth Technical Manual 40-4162700 |  |  |  |  |
|  |  |  |  |  |  |  | When applicable, risk assessment to be developed, accepted and briefed to personnel for new or change to task(s). |  |  |  |  |
| HAUV Operation (tethered mode) |  |  |  |  |  |  |  |  |  |  |  |
| 19 | Piloting of HAUV underwater with tether attached | Potential for impact and damage due to underwater obstacles | Collision leading to damage to HAUV, subsea assets and equipment | 4 | 3 | 12 | Competent HAUV Pilot and team provided for work scope duration | 4 | 1 | 4 | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Survey areas subject to prior inspection and findings identified and communicated |  |  |  | Managed by FADFAE (Client) |
|  |  |  |  |  |  |  | Check and confirm HAUV obstacle avoidance sonar sensors operational |  |  |  | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Check and utilise forward scanning sonar |  |  |  |  |
|  |  |  |  |  |  |  | Radio comms available and include Vessel Master for identified obstacles. Note to checked and confirmed satisfactory prior to operations commencing. |  |  |  |  |
|  |  |  |  |  |  |  | Tether attached for HAUV retrieval and control |  |  |  |  |
|  |  | Potential for impact with surface vessels | Collision leading to damage with other vessels and potential personal injury or fatality | 5 | 3 | 15 | Competent HAUV Pilot and team provided for trials duration | 5 | 1 | 5 | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Visual check and radio comms implemented for project duration with HAUV Operator and Vessel Bridge. |  |  |  | Managed by Vessel Master |
|  |  |  |  |  |  |  | Daily liaison with Vessel Master / Client regarding HAUV planned movements, including tether deployment. |  |  |  | Managed by SHELL / FADFAE (Client) |
|  |  |  |  |  |  |  | Notice to Mariners issued and distributed to applicable organisations / parties |  |  |  |  |
|  |  |  |  |  |  |  | Chase Vessel available to intervene with unauthorised vessel interfaces |  |  |  |  |
|  |  |  |  |  |  |  | Escort vessels navigation and intervention in accordance with the Project Security Plan |  |  |  |  |
|  |  |  |  |  |  |  | Escalation of vessel intervention to be implemented if required |  |  |  |  |
|  |  | Potential for poor surface visibility of HAUV | Collision leading to damage with other vessels and potential for personal injury or fatality | 5 | 3 | 15 | Competent HAUV Pilot and team provided for work scope duration | 5 | 1 | 5 | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | Initial wet test performed prior to work scope and performance accepted by HAUV Superintendent |  |  |  |  |
|  |  |  |  |  |  |  | HAUV set-up for automatic strobe activation |  |  |  |  |
|  |  |  |  |  |  |  | HAUV set up for automatic iridium GPS signalling |  |  |  |  |
|  |  |  |  |  |  |  | Availability of vessel watch alert any other vessel entering area of concern |  |  |  | Managed by Vessel Master (under control of FADFAE - Client) |
|  |  | Potential to lose control of HAUV due to excessive sea state and/or currents | Collision leading to damage with other vessels / obstacles potential for personal injury or fatality | 5 | 3 | 15 | Calibrated current meters submerged from vessel position. Access to meter readings, current and tidal charts. Note benchmark data collated for utilisation for project duration. | 5 | 1 | 5 | Managed by Vessel Master (under control of FADFAE - Client) |
|  |  |  |  |  |  |  | Decision to operate HAUV3 shall be evaluated based on the conditions at the time and how the vessel is behaving in that scenario. Note decision to be taken by the HAUV Superintendent with consultation of the HAUV Supervisor. |  |  |  | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | In all cases, the safe limit to launch, recover or operate the HAUV shall be judged by the HAUV3 Superintendent on a case-by-case basis. |  |  |  |  |
|  |  |  |  |  |  |  | Decision to remain on standby due to sea state, current and/or visibility evaluated beyond operational limitations shall be taken by the HAUV Superintendent with consultation from the HAUV Supervisor |  |  |  |  |
|  |  |  |  |  |  |  | Decision for vessel operational capability which shall affect the HAUV operations shall be taken by the Vessel Master |  |  |  | Managed by Vessel Master (under control of FADFAE - Client) |
| HAUV Emergency Recovery |  |  |  |  |  |  |  |  |  |  |  |
| 20 | Dead Sub Recovery | Potential for loss of HAUV due to immobilisation or tether detachment | Uncontrolled / unseen obstacle leading to damage with other vessels | 5 | 3 | 15 | HAUV set for positive buoyancy which results in slow-release resurfacing | 5 | 1 | 5 | Facilitated by HAUV Supervisor |
|  |  |  |  |  |  |  | HAUV set-up for automatic strobe activation |  |  |  |  |
|  |  |  |  |  |  |  | HAUV set up for automatic iridium GPS signalling |  |  |  |  |
|  |  |  |  |  |  |  | HAUV set up to light configuration |  |  |  |  |
|  |  |  |  |  |  |  | Immediate contact with the Vessel Master to advise / liaise accordingly. |  |  |  |  |
|  |  |  |  |  |  |  | If possible, HAUV Pilot to navigate return to vessel for recovery |  |  |  |  |
|  |  |  |  |  |  |  | If possible HAUV Pilot to navigate return and dock to subsea Garage |  |  |  |  |
|  |  |  |  |  |  |  | If possible HAUV Pilot to navigate to seabed for further recovery |  |  |  |  |
|  |  |  |  |  |  |  | Vessel installed FRC available for deployment in HAUV emergency scenario (in addition to personnel rescue / medivac) |  |  |  |  |

| RA Actions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Action | Actioned by | Target Date | # Date Closed | # Confirmed By |  |

| RA Acceptance |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Accepted |  |  |  | MODUS Manager / Supervisor | Position | Signature | Date |  |
| Yes ☐ |  | No ☐ |  | Anthony Brown | Project Manager |  |  |  |
| Accepted |  |  |  | Client / Third Party | Position | Signature | Date |  |
| Yes ☐ | No ☐ |  | N/A ☐ |  |  |  |  |  |