| HAUV3 Deployment Checklist |  |  |  |
| --- | --- | --- | --- |
| Item | Task | Responsible Person |
| 1. | TBT with HAUV Supervisor, HAUV Control, Bridge, Survey, Deck Supervisor and LARS Winch operator HAUV Pre-dive checklist completed PTW raised and in force Communications check between all parties | HAUV Supervisor Client Rep |
| 2. | Vessel positioned and stationary at the launch position, as previously discussed and displayed on the survey navigation system | HAUV Supervisor Bridge & Survey |
| 3. | Get all clear from Bridge for over boarding garage/vehicle | HAUV Supervisor Bridge |
| 4. | Garage to be latched into snubber and lifted overboard using the LARS. Once fully overboard the snubber will be rotated 90 degrees, so the garage and HAUV are pointing vessel forward. | HAUV Supervisor LARS operator |
| 5. | Garage & HAUV lowered to seabed, while monitoring vehicle heading. Altitude to be monitored and relayed to LARS operator during lowering | HAUV Supervisor HAUV Control LARS operator |
| 6. | HAUV is flown out of the garage, Pilot to confirm they are clear of the garage, in a safe location and tether is clear | HAUV Supervisor HAUV Control |
| 7. | Get all clear from Bridge for recovery | HAUV Supervisor Bridge |
| 8. | Garage recovered to surface and latched into the LARS snubber Survey and HAUV Control to monitor garage heading using the USBL beacons mounted garage aft | HAUV Supervisor HAUV Control LARS operator |
| 9. | Rotate the snubber as required to align the garage for inboard recovery | HAUV Supervisor LARS operator |
| 10. | Garage brought inboard, landed and secured on deck | HAUV Supervisor LARS operator |
| Task Complete |  |  |

| HAUV3 Recovery Checklist |  |  |  |
| --- | --- | --- | --- |
| Item | Task | Responsible Person |
| 1. | TBT with HAUV Supervisor, HAUV Control, Bridge, Survey, Deck Supervisor and LARS operator HAUV Pre-dive checklist completed PTW raised and in force Communications check between all parties | HAUV Supervisor Client Rep |
| 2. | Vessel positioned and stationary at the recovery position, as previously discussed and displayed on the survey navigation system | HAUV Supervisor Bridge & Survey |
| 3. | Get all clear from Bridge for over boarding garage | HAUV Supervisor Bridge |
| 4. | HAUV is flown to a safe location, Pilot to confirm they are clear of the intended landing location and the tether is safe | HAUV Supervisor HAUV Control |
| 5. | Garage to be latched into snubber and lifted overboard using the LARS. Once fully overboard the snubber will be rotated 90 degrees, so the garage is pointing vessel forward. | HAUV Supervisor HAUV Control LARS operator |
| 6. | Garage to be lowered to seabed, Survey and HAUV Control to monitor garage heading using the USBL beacons mounted to the garage aft | HAUV Supervisor HAUV Control LARS operator & Survey |
| 7. | HAUV is flown into the garage, Pilot to confirm they are securely latched and clear for recovery | HAUV Supervisor HAUV Control |
| 8. | Get all clear from Bridge for recovery | HAUV Supervisor HAUV Control |
| 9. | Garage & HAUV recovered to surface and latched into the LARS snubber HAUV Control to monitor garage/vehicle heading during recovery | HAUV Supervisor LARS operator |
| 10. | Rotate the snubber as required to align the garage for inboard recovery | HAUV Supervisor LARS operator |
| 11. | Garage/vehicle brought inboard, landed and secured on deck | HAUV Supervisor LARS operator |
| Task Complete |  |  |