| HAUV3 Pre & Post Dive Physical Checklist Checklist |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project: |  | Dive No: |  | Date: |  |  | Time: |  |
| Operator: |  |  |  | Accepted by: |  |  |  |  |

Tools Required

| The following tools will be required: |  |
| --- | --- |
| - Diaphragm puller tool (SM9 Thruster) (mod-12273) - Diaphragm puller tool (Tilt motor) (MOD-12245) - Fresh water hose (Post Dive only) | - Deck communications - Sonardyne i-Wand - Ulyser PT 9-Ninety tester |

Physical Checks

| WARNING - POSSIBILITY OF FATAL ELECTRIC SHOCK AND DANGER TO PERSONNEL AND EQUIPMENT: MAKE SURE THE PSU IS NOT SWITCHED ON AND THE START KEY IS NOT INSERTED. |  |  |  |
| --- | --- | --- | --- |
|  | Pre-Dive | Post-Dive | Comments |
| Caution Risk of damage to equipment. Fit / Remove Start Key | ☐ | N/A |  |
| Log battery state of charge & charge if necessary | % | % |  |
| Record Battery Voltage Levels | ☐ | ☐ | Stbd: Pt Fwd: Pt Aft: |
| Thrusters – Ensure HV is off and thrusters are not enabled - Check propellors are secure and nozzles are fitted correctly. - Check thrusters are free from debris. - Check Thruster oil levels using the diaphragm puller tool. Ensure that at least one hole is visible on the puller tool. - Rotate propellers several turns. |  | External Checks |  |
|  | ☐ | ☐ |  |
|  | ☐ | ☐ |  |
|  | ☐ | ☐ |  |
|  | ☐ | ☐ |  |
| Check Tilt Unit oil level using the diaphragm puller tool. Ensure that at least one hole is visible on the puller tool. | ☐ | ☐ |  |

|  | Pre-Dive | Post-Dive | Comments |  |
| --- | --- | --- | --- | --- |
| Transformer and Tether checks |  |  |  |  |
| - Check transformer oil level using the diaphragm puller | ☐ | ☐ |  |  |
| - Ensure Fibre Optic Connector O-Ring is present & in good order | ☐ | N/A |  |  |
| - Check Tether Chinese finger/Cortland grip is secure | ☐ | ☐ |  |  |
| Check that all cameras are correctly fitted and secure, remove lens caps and clean lenses with soapy water. | ☐ | ☐ |  |  |
| Check all lights are correctly fitted and free from damage. | ☐ | ☐ |  |  |
| Check air bleed screws are tightened: |  |  |  |  |
| - EPOD | ☐ | ☐ |  |  |
| - Antenna Pod | ☐ | ☐ |  |  |
| Confirm Bulkheads and Harnesses are secure and ensure blanks are fitted where necessary. | ☐ | ☐ |  |  |
| Check all anodes and replace if necessary | ☐ | ☐ |  |  |
| Check for corrosion on all housings, bulkheads, equipment and brackets | ☐ | ☐ |  |  |
| Check that buoyancy blocks and Rear Wing are correctly fitted, secure and not damaged. | ☐ | ☐ |  |  |
| Check additional any equipment is correctly fitted and secure. | ☐ | ☐ |  |
| Test PT9 Ninety with Ulyser tester, approx. 3.5v. | ☐ | N/A |  |
| Check operation of Transponder with iWand ping check. | ☐ | N/A |  |
| Fit recovery strop: ensure cable ties are part snipped. | ☐ | N/A |  |
| Fit the deployment hook and insert the release pin. (if applicable) | ☐ | N/A |  |
| Set recovery poles with carabiners held open, cable tie’s part snipped. | ☐ | N/A |  |
| Rinse AUV with fresh water | N/A | ☐ |  |

Physical Checks

|  | Pre-Dive | Post-Dive | Comments |
| --- | --- | --- | --- |
| Confirm the HMI displays all relevant screens for operations |  |  |  |
| Confirm Emergency Mission is set and appropriate | ☐ | N/A |  |  |
| Check correct versions of SAC, RAC and PAC are loaded | ☐ | N/A |  |  |
| Confirm that RAC PAC & SAC have sufficient free space: | ☐ | N/A |  |
| Check with surveyor to run RoboCopy and delete old log files | N/A | ☐ |  |
| Test Wi-Fi Comms to HAUV. | ☐ | N/A |  |
| Test Fibre Comms to HAUV | ☐ | N/A |  |
| Test Radio Comms to HAUV (if applicable). | ☐ | N/A |  |
| Test the survey/sensor suite: |  |  |  |
| - FiGS system (switch off o/c) | ☐ | N/A |  |
| - TSS660 system | ☐ | N/A |  |
| - Edgetech SSS (switch off o/c) | ☐ | N/A |  |
| - SBP | ☐ | N/A |  |
| - Gemini MBES (switch off o/c) | ☐ | N/A |  |
| Test thrusters, checking for correct function and orientation: |  |  |  |
| - OCB | ☐ | N/A |  |
| - POCB | ☐ | N/A |  |
| Check Lamps operation / Lamps off | ☐ | ☐ |  |
| Check Obstacle Avoidance Sonar (HMI to Thrust mode) / OAS off | ☐ | ☐ |  |
| Check the camera tilt unit operation and is free from fouling. | ☐ | ☐ |  |
| SprintNav (Gyro): Operational | ☐ | N/A |  |
| DGPS: |  |  |  |
| - Check Operational | ☐ | N/A |  |
| - Check Corrections Requested | ☐ | N/A |  |
| - Check Beam assigned in Novatel | ☐ | N/A | Assign Appropriate Beam |

Control Cabin Desk/Survey Checks

Checklist

|  | Pre-Dive | Post-Dive | Comments |
| --- | --- | --- | --- |
| Trigger Board – Settings verified & Trigger Enabled |  |  | Settings |
| - DVL | ☐ | N/A |  |
| - MBES | ☐ | N/A |  |
| - SSS/SBP (if applicable) | ☐ | N/A |  |
| DVL – On & Triggered | ☐ | N/A |  |
| Depth Sensor Zeros on Navigation Tab | ☐ | N/A |  |
| UV-SVP/SVS Zeros on Navigation Tab | ☐ | N/A |  |

Control Cabin Desk/Survey Checks (Cont’d)

Checklist

Winch (if Applicable)

|  | Pre-Dive | Post-Dive | Comments |
| --- | --- | --- | --- |
| Winch pay out / pay in. | ☐ | N/A |  |
| Check that guide tubes are greased. | N/A | ☐ |  |
| Cooling water turned on / off. | ☐ | ☐ |  |
| Rinse drum with fresh water | N/A | ☐ |  |

|  | Pre-Dive | Post-Dive | Comments |
| --- | --- | --- | --- |
| Turn On / Off strobe and confirm operations (If applicable). | ☐ | ☐ |  |
| Remove / Refit the Iridium magnet (Confirm Emails) | ☐ | ☐ |  |
| SprintNav Status – Navigation Mode | ☐ | N/A |  |
| IXSEA Logger – Set Logging (If applicable) | ☐ | N/A |  |
| Low Voltage: |  |  |  |
| - Lights Enabled | ☐ | N/A |  |
| - Tilt Unit Enabled | ☐ | N/A |  |
| - Cameras Enabled | ☐ | N/A |  |
| - Video Enabled | ☐ | N/A |  |
| - OAS Enabled | ☐ | N/A |  |
| High Voltage WARNING - POSSIBILITY OF FATAL ELECTRIC SHOCK AND DANGER TO PERSONNEL AND EQUIPMENT: CORRECT PPE IS TO BE WORN WHEN HANDLING THE TETHER WITH HV SUPPLIES SWITCHED ON |  |  |  |
| Thrusters Enabled | ☐ | N/A |  |
| Deck Mode Selected | ☐ | N/A |  |
| Thruster Enable Selected | ☐ | N/A |  |
| Zero Pitch & Roll Selected | ☐ | N/A |  |

Splash Zone/Water Borne Checks

Off Deck Checks

|  | Pre-Dive | Post-Dive | Comments |
| --- | --- | --- | --- |
| SprintNav Just prior to going subsea, turn off GPS input | ☐ | N/A |  |
| OAS Operational | ☐ | N/A |  |
| Depth Calibration | ☐ | N/A |  |
| - Deck Mode Selected | ☐ | N/A |  |
| - Settings DEPTH CAL AT SURFACE Selected | ☐ | N/A |  |
| DVL Operational (Launch type dependent) | ☐ | N/A |  |
| Depth Sensor Meaningful values on Navigation tab | ☐ | N/A |  |
| UV-SVP/SVS Meaningful values on Navigation tab | ☐ | N/A |  |
| FiGS Meaningful values | ☐ | N/A |  |
| Edgetech Image correct (Launch method dependent) | ☐ | N/A |  |
| Gemini Image correct (Launch method dependent) | ☐ | N/A |  |

NOTES FOR ABOVE CHECKS: